Fault codes for Bosch and Simtec Mpi main engine ECU

The warning lamp will illuminate when the ignition is turned on, but will go out as soon as the engine is started. It will, however, stay on if the ecu detects a fault with the engine. If it is a serious fault the ecu will switch to an emergency programme, and allow the vehicle to be driven to a garage for repair.

To activate fault codes, connect wire A and B of the 10 pin diagnostic plug. Fault codes will display as flashes - with code 12 being 1 flash, 13 being 2 and so on. To erase the fault code, turn the ignition on/off 25 times. Otherwise disconnect the battery earth lead for at least a minute. This will, however, erase the memory from other electronic units

| CODE | FAULT |
| --- | --- |
| 12 | Diagnosis initiated |
| 13 | Lambda Sensor (Open circuit) |
| 14 & 15 | Coolant temperature sensor (Voltage Low & High respectively) |
| 16 & 17 | Knock Sensor 1&2 respectivley. (No voltage charge) |
| 18 | Knock control unit (no signal) |
| 19 | Crankshaft sensor (incorrect signal) |
| 21 & 22 | Throttle position sensor (voltage high & low respectively) |
| 23 | Knock control module |
| 24 | Vehicle speed sensor (no signal) |
| 25 - 29 | Injector valve 1-5 respectively (voltage high) |
| 31 | Crankshaft sensor (no signal) |
| 32 | Injector valve 6 (voltage high) |
| 33 & 34 | EGR valve (voltage low and high respectively) |
| 35 | Idle speed control valve |
| 37 | Fault code light (voltage low) |
| 38 & 39 | Lambda senor (voltage low and high respectively) |
| 41 & 42 | Vehicle speed sensor (voltage low and high respectively) |
| 44 & 45 | Lambda sensor (weak and rich mixture respectively) |
| 46 & 47 | Air pump relay (voltage high and low respectively) |
| 48 & 49 | Battery voltage (low and high respectively) |
| 51 | Electronic control unit (hardware failure) |
| 52 | Fault code light (voltage high) |
| 53 & 54 | Fuel pump relay (voltage low and high respectively) |
| 55 | Electronic control unit (hardware failure) |
| 56 & 57 | Idle air control (voltage low and high respectively) |
| 59 | Inlet manifold valve 1 (voltage high) |
| 61 & 62 | Fuel tank vent valve (voltage low and high respectively) |
| 63 | Inlet manifold valve 1 (voltage high) |
| 65 & 66 | Idle co potentiometer (voltage low and high respectively) |
| 67 | Throttle valve switch |
| 68 | Idle speed switch (not opening) |
| 69 & 71 | Air temperature sensor (voltage low and high respectively) |
| 72 | Throttle valve switch (not opening) |
| 73 & 74 | Air flow sensor (voltage low & high respectively) |
| 75 | Air transmission torque control (voltage low) |
| 76 | Air transmission torque control (Ignition retard long) |
| 79 | Traction control (Incorrect ignition/igntion cutoff) |
| 81-86 | Injector valve 1 - 6 (voltage low) |
| 87 & 88 | Air conditioning cutoff relay (voltage low) |
| 89 & 91 | Lambda sensor heater (voltage low and high respectively) |
| 92 | Camshaft sensor (incorrect signal) |
| 93 & 94 | Hall effect sender (voltage low and high respectively) |
| 97 | Traction control (incorrect signal) |
| 98 | Lambda sensor (open circuit) |
| 113 | Boost control (out of range) |
| 114 | Boost pressure idle (above upper limit) |
| 115 | Boost pressure full (below upper limit) |
| 116 | Boost pressure (above upper limit) |
| 117 & 118 | Waste gate valve (voltage low and high respectively) |
| 121 | Lambda sensor 2 (weak mixture) |
| 122 | Lambda sensor 2 (rich mixture) |
| 123 | Inlet manifold 1 (blocked) |
| 124 | Inlet manifold 2 (blocked) |
| 132 | EGR valve (Incorrect signal) |
| 133 & 134 | EGR valve 2 (voltage high and low respectively) |
| 135 | Fault code light |
| 136 & 137 | Electronic control unit |
| 141 | Secondary air pump (not enough air) |
| 142 | Secondary air pump (air without pump) |
| 143 | Immobiliser function in ECU (no/incorrect signal) |
| 144 | No immobiliser signal received |
| 145 | Incorrect immobiliser signal received |